# SOPs — PVS ClearView

Window cleaning, gutter cleaning, pressure washing, soft washing, junk removal. Read alongside the company-wide SOPs.

## Service standards

A finished ClearView visit means:

- All scope items completed per the work order

- No streaks, smudges, or fingerprints on glass we touched

- Sills wiped and dry

- Gutters: debris removed by hand, downspouts flushed clear

- Pressure / soft wash: surface uniformly clean, plantings rinsed, no detergent residue

- All ladders, hoses, equipment loaded back in the truck

- Customer entryways unlocked the way we found them

## Pre-route checklist (every morning)

- Squeegees clean, rubber in good shape (rotate worn ones)

- Microfibre cloths (clean, dry, divided by surface)

- Window-cleaning solution mixed and labeled

- Bucket and applicator

- Extension pole (water-fed for upper floors if booked)

- Ladders sized for the job (8' + extension, both)

- Pressure washer fueled if booked

- Soft-wash applicator + biodegradable detergent if booked

- Gloves, eye protection, gutter scoop, garbage bags

- Tarp for catching debris under gutters

## Window cleaning procedure

Interior:

- Move or cover anything below the window (electronics, soft furniture).

- Spray solution onto microfibre, not the window. Wipe sills and frame to remove grit before squeegee.

- Squeegee top to bottom in overlapping passes.

- Detail edges with a dry microfibre.

- Wipe sill dry.

- Replace anything you moved.

Exterior:

- Walk and inspect frames before starting — note any damaged seals or cracked glass and flag to the customer.

- Pre-rinse if heavily soiled (pollen, bird droppings).

- Apply solution with applicator pole or sprayer.

- Squeegee or water-fed brush on second-storey + up.

- Detail edges.

- Final rinse if water-fed (deionised water leaves no spots).

Quality check: Stand back 6 feet, look at every window. Streak = re-do. The streak-free guarantee is the brand — it's not negotiable.

## Gutter cleaning procedure

- Tarp the ground below the gutter run.

- Ladder set per ladder safety (1:4 ratio, top extending 3 rungs above contact point, tied off where possible).

- Hand-scoop debris into a bucket. Don't drop debris from the ladder — it splatters and ruins the rinse later.

- Flush every downspout from the top with the hose. Confirm flow at the bottom. If blocked, snake or remove the elbow to clear.

- Visual-inspect every gutter section for: loose fasteners, sagging, sealant gaps, rust holes, downspout damage.

- Note repairs needed → photo + flag the customer with a free quote for the fix.

- Rinse the gutters from the outside if heavily stained (separate service — quote, don't include free).

- Pack out all debris in bags. Nothing in customer's bin without permission.

## Pressure washing procedure

- ALWAYS test in a hidden corner first — different surfaces handle different pressure

- Driveway / concrete: 2500–3000 PSI, surface cleaner attachment

- Wood deck: soft wash or low-pressure (≤ 1200 PSI), wide fan tip, wash with the grain

- Brick / paver: medium pressure, surface cleaner for flat runs

- Sand-set pavers will lose jointing sand — quote re-sand if customer wants the polished look

- Always pre-rinse plantings. Cover with tarp if detergent will hit.

- Post-rinse plantings again

## Soft-wash (house wash) procedure

- For siding, soffit, stucco

- Low pressure + biodegradable detergent (manufacturer-rated for exterior)

- Apply bottom up (prevents streaks), rinse top down

- Dwell time: 5–10 minutes; don't let it dry on the surface

- Watch for wind drift onto cars, neighbours, plants

- Final walkaround — look for missed spots in raking light

## Junk removal / property cleanout procedure

- Quote in person when possible. Photos for remote quotes.

- Confirm scope before loading: what stays, what goes.

- Separate as you load:

  - Donations (Goodwill, Salvation Army, local thrift)

  - Recyclables (metal, e-waste, cardboard)

  - Landfill

- Treat the property like our own — no scuffs on walls, no marks on floors. Use moving blankets.

- Hazardous items (paint, propane, batteries, chemicals) → DO NOT accept. Flag the customer to municipal hazardous waste day.

- End-of-job photo of the cleared space.

- Itemised donation/recycling receipt to the customer if they want it for tax purposes.

## Safety standards (ClearView is highest-risk division)

- Ladders: 3-point contact at all times. No reaching beyond your belt buckle line. Tie off when possible.

- Heights over 1 storey: spotter required at the base.

- Roof access: approved roof shoes only, never on wet/icy roof. Harness + tie-off on any roof over 4/12 pitch.

- Pressure washer: never aim at people, animals, electrical, or windows.

- Electrical: treat all overhead lines and wet receptacles as live. 10-foot clearance from any line.

- Solo work prohibited for any job involving an extension ladder or a roof.

## Quality red flags (stop and call Office)

- Cracked or compromised window glass — don't clean, don't touch, photo

- Loose gutter pulling away from fascia — risk of collapse

- Soft / rotted fascia or soffit

- Active wasp / hornet nest in gutter or eaves

- Power line within 10 feet of work area

- Roof too steep / icy / slippery for safe access