# Training — PVS ClearView

ClearView is the highest-skill, highest-risk division. Window cleaning, gutter work, pressure washing, soft washing — all customer-visible, all requiring ladder work. Take training seriously.

## Week 1 — Safety + ground-level techniques

### Day 1 — Orientation + ladder safety

- Read Vision + Org Structure + General SOPs + ClearView SOPs

- Ladder safety: 1:4 angle, 3-point contact, 3 rungs above contact point, never reach past your belt buckle, tie-off

- Practice setting an extension ladder against the shop wall — 10 reps until set-up is automatic

- Sign safety attestation

### Day 2 — Window cleaning fundamentals (interior)

- Spray microfibre, not glass

- Squeegee technique: top-to-bottom, overlapping passes

- Detail edges with dry microfibre

- Streak inspection: stand back 6 feet, raking light

- Practice on the shop windows until streak-free is consistent

### Day 3 — Exterior windows, ground floor only

- Same technique outside, ground level only

- Pre-rinse heavily soiled windows

- Water-fed pole intro (no climbing yet)

- Customer-handling drill: greeting at the door

### Day 4 — Gutter basics

- On the ground: tarp set-up, debris bagging

- Hose flush technique: top down, confirm flow at outlet

- Inspection checklist: fasteners, sag, sealant, rust

- Practice on the shop building with crew lead spotting

### Day 5 — Ladder day

- Step ladder work on first-floor windows + low gutters

- Extension ladder under supervision only

- Crew lead spots every climb

- End-of-week ladder safety quiz

## Week 2 — Live work + pressure washing

### Days 6–8 — Live window + gutter visits

- Shadow + assist on real customer properties

- Helper does the ground floor, crew lead does the upper

- Photo and handoff at every property

### Day 9 — Pressure washing introduction

- Test patch on shop driveway

- Surface cleaner attachment on flat runs

- Soft-wash detergent application

- Plant rinse protocol

### Day 10 — Solo run on small property

- Helper runs a small (1-story, low gutter, 8 windows) job solo with crew lead drive-by QC

- Debrief, sign-off

## Streak-free guarantee — the brand promise

This is the most important quality standard in ClearView. Drill it in.

What counts as a streak:

- Visible line, smudge, or dried-water mark on the glass

- Edge film where the squeegee didn't reach

- Sill water marks the customer can see

How to never leave one:

- Clean glass first, sill last

- Squeegee in overlapping passes (overlap 1 inch)

- Wipe the squeegee blade after every pass

- Detail the edges with a dry, clean microfibre

- Final check: walk 6 feet back, look from multiple angles, ESPECIALLY the angle the customer looks from (often the kitchen sink or living room couch)

If you spot a streak after-the-fact: go back, do it again, no charge. The guarantee is the brand.

## Pressure vs. soft wash — when each is appropriate

| Surface | Method | Pressure | Notes |
| --- | --- | --- | --- |
| Concrete driveway | Pressure + surface cleaner | 2500–3000 PSI | Test patch always |
| Pavers | Pressure + surface cleaner | 2000 PSI | Quote re-sanding |
| Brick | Medium pressure | 1500–2000 PSI | Watch for old mortar |
| Wood deck | Soft wash or low pressure | ≤ 1200 PSI | With grain only |
| Vinyl / aluminum siding | Soft wash | ≤ 500 PSI | Bottom-up apply, top-down rinse |
| Soffit / fascia | Soft wash | ≤ 500 PSI | Watch wind drift |
| Stucco | Soft wash only | ≤ 300 PSI | Test patch, no high pressure ever |

When in doubt — soft wash. You can always go up in pressure; you can't undo damage.

## Ladder + height safety quiz (must pass before independent height work)

- What angle is a safe ladder placement? (1 foot out for every 4 feet up)

- How far above the contact point should the ladder extend? (3 rungs)

- What's the maximum distance you can reach to either side without moving the ladder? (Belt buckle should never cross outside the rails)

- When is solo ladder work allowed? (Never. ClearView ladder work requires a spotter.)

- What weather conditions stop ladder work immediately? (Wind > 25 km/h, rain, ice, lightning visible)

- What's the clearance distance from any overhead power line? (10 feet — assume all lines are live)

Crew lead administers verbally on Day 5. Must score 100% to advance.

## Customer-handling drills

### Arrival

"Good morning, I'm {name} from Prestige View Services — we're here for your window cleaning. I'm going to walk the property quickly to get my bearings — anything you'd like me to know before I start?"

### Mid-job customer check

(If the customer's home and the job is long)

"Just wanted to let you know we're about halfway — the upper floor is next. Should take another 45 minutes."

### Streak callback

"Thanks for letting me know — I'll come right back. Streak-free is our guarantee, no charge to make it right."

### Spot a problem on the home (rotted fascia, etc.)

"While I was up there I noticed {issue}. I'll have the office send you a quote so you can decide — it's not urgent but worth knowing."

### Customer wants a discount

"Our pricing's the same for every customer in your neighbourhood — we don't change quotes after a job. What I can do is bundle this with your next gutter clean to save on the trip. Want me to ask the office to put together a bundle quote?"

## Sign-off

By end of Week 2, the helper signs off on:

- Read and understood Vision + Org + General SOPs + ClearView SOPs

- Passed ladder + height safety quiz at 100%

- Demonstrated streak-free window cleaning interior + exterior

- Completed first solo small ClearView job to quality standard

- Knows when to recommend pressure vs. soft wash

Crew lead signs the other side. File copy in HR folder.

## 30-day check-in

Same template as LawnPros. Add specifically:

- Is ladder work feeling natural? Any height concerns?

- Have you had a streak callback yet? What did you learn?

- Are you confident running the pressure washer solo?