# Training — PVS SnowLand

Snow work is short-window, high-stress, weather-dependent. Mistakes happen at 4am in low visibility. Training has to compensate.

## Pre-season classroom (October — 1 full day)

All SnowLand crew, returning or new, do this every year before the first storm.

### Morning — Route + equipment

- Review the season's customer list

- Walk every truck: plow, salt spreader, shovels, brooms, ice chippers, headlamps, first-aid

- Hands-on plow attachment + detachment drill (each crew member)

- Practice driveway approach in an empty lot:

  - Reading the lay before pushing

  - Pile location (away from sight lines, never on lawn beds)

  - Garage door stop (24" clear)

  - Mailbox / marker awareness

### Afternoon — Safety, communication, customer expectations

- Storm activation protocol

- Mid-storm pace expectations

- Final-pass standards

- Salt application: light, even, not on plantings

- Incident reporting (slip-and-fall liability awareness)

- Customer call handling — what Office handles, what crew handles

End of day: signed seasonal safety attestation, equipment readiness sign-off.

## First-storm shadow (returning crew + new helpers)

- New helpers ride passenger seat for the first 2 storms

- They watch route execution, learn the customer-by-customer notes (this driveway is steep, this one has a tricky turn, this customer wants the side walkway done, etc.)

- After 2 storms, they take half the route under crew lead supervision

## Plowing technique standards

### Approach

- Slow into the driveway. Read it before you push.

- First pass down the centre, then sides outward.

- Push snow to the side opposite the front door entrance.

- Never push across the road. Never into a neighbour's lawn.

- Watch for: driveway markers, garage door, low shrubs, sprinkler heads, paving stone borders.

### Pile placement

- Far corner of the driveway, opposite the door

- Never blocking sight lines at the road

- Never against the garage door

- Never on lawn beds (kills turf in spring — extra liability cost)

### Garage door rule

- Stop the plow 24" short of the garage door

- Shovel the rest by hand — even if it adds 30 seconds

- Plow scratches on paint cost more than the time savings

## Walkway clearing standards

- Curb-to-door minimum

- One shovel-width on standard plan, two on premium

- Clear all steps fully — top, treads, sides

- Mailbox: 3-foot box cleared around it (so the carrier can reach)

- Garbage day: clear pickup area

## Salt / ice control standards

- Apply lightly. Salt is corrosive and over-application damages concrete + kills plants.

- Standard plan: spot-treat icy patches only

- Premium plan: full driveway and walkway light application

- NEVER salt onto:

  - Coloured / stamped concrete (stains)

  - Garden bed perimeters (plant damage)

  - Steel railings (rusts hardware)

- Always wear gloves + eye protection when handling

## Driving rules during a storm

- Speed limit on residential routes: 30 km/h max

- All lights on (headlights, plow lights, flashers as appropriate)

- 4WD engaged before entering any driveway

- Phone in dash mount — no handheld dispatch during plow operation

- If visibility drops below 50 m: pull over, wait

## Customer-handling drills

### Customer flags down the truck mid-route

"Hi — you're in the route, we'll be at your place within the window. If something's urgent, please call the office now and they'll prioritise."

### Customer complains about pile location

"Got it — I'll move it on the next pass. Any spot in particular that works for you?"

### Customer asks for off-contract walkway

"I can do it today as an add-on — I'll call the office for a quote so you know the price before I start. Takes 2 minutes."

### Customer claims damage

"I want to take a look right now and get a photo. Don't worry — if we caused it, we own it. Let me call the office and we'll get a follow-up scheduled."

NEVER admit fault on the spot. Document, escalate, let the Owner handle the resolution.

## Storm wrap-up training drill

After every storm, debrief with the new helper:

- What did you see? What's tricky on this route?

- Did anything surprise you?

- Any equipment issue?

- Any customer interaction you want to walk through?

Capture in a notebook. Patterns across drivers become SOP updates.

## Cold + fatigue management

Snow shifts can run 12+ hours during heavy storms. Crew leads must:

- Mandate a 15-minute break every 3 hours (heated truck)

- Rotate plow + shovel roles

- Watch for signs of cold stress: shivering that won't stop, slurred speech, slow reaction time → END THE SHIFT

- Eat + hydrate every 4 hours — running on coffee alone causes mistakes

If a crew member can't continue safely, the Owner is called and a replacement rotates in. No exceptions.

## Sign-off (first storm complete)

After a new helper has run a full storm cycle with the crew lead:

- Read and understood Vision + Org + General SOPs + SnowLand SOPs

- Passed pre-season classroom safety attestation

- Demonstrated safe plow approach and pile placement

- Demonstrated walkway + salt application standards

- Successfully handled at least one in-person customer interaction

- No safety incidents during the shadow phase

Crew lead signs. File copy in HR.

## End-of-season debrief (April)

All SnowLand crew, 90 minutes:

- Total storms run, total hours

- Customer feedback themes (good and bad)

- Equipment wins and losses (what's holding up, what to replace)

- Routes that need re-balancing for next year

- Crew rotation into spring divisions

Notes → updates to next October's pre-season classroom.